SADP

Course: BTECH (CS01)

Set: A  |  Time: 3 Hours  |  Max Marks: 35  |  Date: 15-03-2026

________________________________________________________________________________

GENERAL INSTRUCTIONS: Write all the questions

| Name: ____________________ | Roll No: ________________ |
| --- | --- |
| Student Sign: ____________ | Teacher Sign: ____________ |

SECTION A: MCQs

1.  	[2 Marks]

Which of the following is a core characteristic of an embedded system?

a) It is designed for general-purpose computing tasks.

b) It performs a specific, dedicated function within a larger system.

c) It requires a large amount of user-replaceable hardware.

d) It always uses a high-performance graphical user interface.

2.  	[2 Marks]

In an embedded system, what is the primary difference between a microcontroller and a microprocessor?

a) A microcontroller lacks an internal CPU.

b) A microprocessor includes integrated RAM, ROM, and I/O on a single chip.

c) A microcontroller integrates the CPU, memory, and peripherals on a single chip.

d) Microprocessors are only used for real-time applications.

3.  	[2 Marks]

Which communication protocol uses only two wires, SDA and SCL, to connect multiple integrated circuits?

a) SPI

b) UART

c) I2C

d) CAN

4.  	[2 Marks]

Which mechanism is used in Real-Time Operating Systems (RTOS) to solve the problem of priority inversion?

a) Context Switching

b) Priority Inheritance Protocol

c) Round Robin Scheduling

d) FIFO Buffer Management

5.  	[2 Marks]

What type of memory is typically used to store the firmware or boot code in an embedded system?

a) RAM

b) ROM or Flash Memory

c) Cache Memory

d) Virtual Memory

SECTION B: SHORT ANSWERS

6.  	[5 Marks]

Define an embedded system and provide two common examples.

7.  	[5 Marks]

Explain the purpose and operation of a Watchdog Timer in embedded hardware.

8.  	[5 Marks]

What is interrupt latency and why is it a critical factor in embedded system design?

9.  	[5 Marks]

Differentiate between Hard Real-Time and Soft Real-Time systems.

10.  	[5 Marks]

Describe the Harvard Architecture and its advantage in embedded processors.

________________________________________________________________________________

--- End of Question Paper ---